Adaptive QoS Routing in SDN with Fuzzy Logic

Team 03

FIRST SEMESTER 2025-26

Sept 07, 2025

UNDER THE SUPERVISION OF

Dr. Virendra Singh Shekhawat

Associate Professor

Department of Computer Science and Information System

TEAM MEMBERS

| Vrudhula Sreedhar | 2025H1030056P |
| --- | --- |
| Jalluri Mahesh Kumar | 2025H1030061P |

Project Proposal:

1. Background

Name of paper

"Adaptive Flow Routing Model in SDN" by Mykola Beshley, Marian Seliuchenko, Oleksiy Panchenko, and Artur Polishuk.

Brief summary of the paper’s problem domain/challenge, goals, and technical approach

Problem Domain/Challenge: Traditional routing protocols like OSPF and EIGRP use relatively static metrics (e.g., bandwidth, delay) and are not flexible enough to manage heterogeneous network traffic where different applications have vastly different Quality of Service (QoS) requirements. In modern networks, real-time traffic (like VoIP or video conferencing) needs low latency and jitter, while bulk data transfers require high bandwidth. A single routing policy is insufficient to optimize paths for all traffic types simultaneously.

Goals: The primary goal of the paper is to develop an adaptive routing mechanism for Software-Defined Networks (SDN) that can handle heterogeneous traffic by considering detailed, up-to-date QoS requirements for different traffic classes as defined by the ITU-T.

Technical Approach: The authors propose a generalized, multi-parameter routing metric, W(i,j)​, to evaluate the cost of a link between two nodes i and j. This metric incorporates five key network parameters:

- Packet Loss Ratio

- Packet Transmission Delay

- Bandwidth

- Jitter

- Computational Efficiency (inverse of load) of the destination node

The novelty of their approach lies in using a set of five weighting coefficients (X1​ to X5​) that can be dynamically adjusted by a central SDN controller. By changing the values of these coefficients, the controller can prioritize certain QoS parameters over others, effectively creating different logical network topologies and optimal routes for different classes of traffic. For instance, for VoIP traffic, the weights for delay and jitter would be set high, while for a file transfer, the weight for bandwidth would be prioritized.

Summary of the paper’s current implementation, evaluation strategy, and key results

Implementation: The authors implemented a simulation model of an SDN network to test their proposed routing algorithm.

Evaluation Strategy:

- Platform: They used Mininet to create a virtual SDN network topology consisting of 18 nodes. Open vSwitch was used for the switches, and the Iperf tool was used to generate heterogeneous TCP and UDP traffic.

- Methodology: They simulated a scenario with high network load and self-similar traffic to create realistic burstiness and congestion. The performance of their proposed adaptive routing algorithm was compared against two standard routing protocols: OSPF and EIGRP.

- Scenarios: They tested their algorithm under three different weighting schemes:

  - Equal weights for all five parameters.

  - Prioritizing delay (a high weight for the delay coefficient).

  - Prioritizing bandwidth (a high weight for the bandwidth coefficient).

Key Results: The proposed adaptive routing model demonstrated significant performance improvements over traditional protocols.

- With equal weights, the model reduced average delay by 33% compared to OSPF and 20% compared to EIGRP.

- When prioritizing delay, the delay reduction was even more substantial: 54% vs. OSPF and 45% vs. EIGRP.

- When prioritizing bandwidth, the packet loss ratio was reduced to nearly 0%, proving its effectiveness for services sensitive to packet loss.

- The study showed that by adjusting the metric's weights, the SDN controller could intelligently and dynamically change traffic routes to meet specific QoS goals.

2. Plan

Proposed implementation (language, framework, etc.)

- Language: Python 3. It is the de-facto standard for SDN controller development and network scripting, with extensive library support.

- Frameworks & Tools:

  - Mininet: To create the virtual network topology, hosts, and switches, replicating the 18-node setup from the paper. This allows for a direct comparison of results.

  - Ryu SDN Controller: A popular, component-based, open-source SDN controller written in Python. We will develop a custom Ryu application to implement the routing logic.

  - Iperf: To generate different classes of traffic (TCP and UDP) and measure key performance indicators like throughput, jitter, and packet loss, just as in the original paper.

Evaluation strategy (testing platform/setup, simulated data/traces, etc.)

- Testing Platform: A Linux environment (either a dedicated machine or a virtual machine) with Mininet + Ryu + Openflow setup.

- Setup:

  - Phase 1 (Reproduction): Recreate the 18-node network topology described in the paper within Mininet.

  - Phase 2 (Extension): Implement the core logic of the paper's adaptive routing metric within a Ryu controller application. This application will monitor network statistics (delay, loss, etc.) from the OpenFlow switches.

  - Data Generation: Use Iperf to generate multiple, concurrent traffic flows that simulate different service classes (e.g., low-latency UDP for VoIP, high-throughput TCP for file transfer). We will simulate self-similar traffic patterns to induce congestion and test the adaptability of the routing logic.

Key results trying to reproduce

The first goal is to validate our implementation by reproducing the key quantitative results from Table 1 in the paper. We will measure and compare the average delay, packet loss percentage, and jitter for our simulated network under the following conditions:

- Baseline performance using a simple routing protocol (e.g., L3 switching).

- The paper's proposed algorithm with equal weighting coefficients for all five metrics.

- The algorithm with a high weight assigned to the delay metric.

- The algorithm with a high weight assigned to the bandwidth metric.

We will also qualitatively verify that the chosen traffic paths change dynamically based on the weighting scheme, as shown in Figure 3 of the paper.

3. Discussion of how you can compare your findings

The original paper successfully demonstrates the benefit of using adaptable, weighted metrics for routing but acknowledges that "the selection of the values of weighting coefficients is not a trivial task." Our proposed extension directly addresses this limitation by automating the selection process using an intelligent mechanism.

New questions/settings trying to evaluate that are not addressed in the original paper

Our core extension is to implement a separate Fuzzy Logic Controller (FLC) for each traffic class within the SDN controller. Instead of manually setting static weights, each FLC will dynamically and continuously adjust the five weighting coefficients (X1​ through X5​) for its specific traffic class based on real-time network conditions.

The inputs to each FLC will be the five measured network parameters (delay, jitter, loss, bandwidth, utilization), and the outputs will be the five adjusted weights for the routing metric. This leads to several new research questions:

- How does an intelligent, adaptive weighting system compare to a static, priority-based one? We can compare the performance of the FLC-driven system against the paper's static high-priority scenarios. The hypothesis is that the FLC will provide better overall performance by making more dynamic adjustments rather than simply maxing out one coefficient's weight.

- What happens if the nature of the traffic flows changes dynamically? We can evaluate scenarios not covered in the original paper:

  - Mixed Flows: What if the network has to handle a mix of short, latency-sensitive flows (e.g., DNS queries, web clicks) and long, bandwidth-intensive flows (e.g., video streams, backups)? We can test whether the separate FLCs for each class can find a better equilibrium, preventing the long flows from starving the short ones.

  - Workload Shifts: We can simulate a workload shift from uniformly distributed traffic to a Zipf distribution, where a few flows become extremely heavy. We will measure how quickly and effectively the FLCs adapt the routing paths to handle this sudden "hotspot" in the network.

By implementing this fuzzy logic extension, this project will not only reproduce the original paper's findings but also build directly upon its future work, exploring a more intelligent and truly adaptive solution for QoS-aware routing in SDN.